崇新学堂

2022－2023学年第一学期

实 验 报 告

课程名称：   Experiments of Introduction to EECS

实验名称：         Turning Heads

专  业  班  级     2021级崇新学堂

学  生  姓  名   葛明烨、余昊、崔宇鑫

实  验  时  间     2022年11月8日

Step1

Vmin=0.53V

In actual measurement, we found that Vmin is 0.2 higher than voltage dropped from high to low. We took a smaller value.

Step2

rm=10.2Ω

Step3

Vmotor=0.01V

Check Yourself 1.

The motor is not running. Vmotor<Vmin, Vmotor is short of the minimum voltage required to make the motor turn.

Step4

Vmotor=0.01V

The unturned motor corresponds to the pure resistance in a circuit.

rm//R2≈10.2Ω

VR1≈9.899V

Because of the disparity between 10.2Ωand 1000Ω, almost all voltage was added into R1.

Check Yourself 2.

The theory matched the measurement in the previous part.No counter electromotive force is generated when the motor is not turned.

Step5

buffered divider circuit:

volltage of pin 5-6

Motor pot alpha

Moto rotational velocity

Step6

VM=4.93V

Motor functions well

Check Yourself 3.

Buffered divider circuit created isolation between motor and basic circuit.It is a good way to get a special point volatge. Equivalent to output a new voltage source. 5V≈4.93V>0.53V Enough to drive the motor.

Step7

Check Yourself 4.

Motor pot alpha

Motor rotational velocity

In the first half of this graph,vm=0, the corresponding voltage is 0(α=0)

In the second half of this graph, vm gradually increase to a fixed value, the velocity of the motor is constant after buffering for a certain time(α=0.1)

Step8

1/4 turn

1/2 turn

3/4 turn

In actual measurement, motor rotational velocity quickly stabilized. The voltage value reflected the motor rotational velocity.

Step9

VX=5V

Check Yourself 5.

We take a 5V node from a 10V node in a simple circuit. It is directly connected to the motor using the buffer.

Step10

Bidirectional Speed Controller circuit in CMax:

Checkoff 2.

Motor pot alpha

Motor rotational velocity

VM

Plus or minus of VM(V5-V6) is relative to motor rotational velocity.

When α=0.25,VM+=2.5V, VM≈-2.5V,the motor rotates inversely.

When α=0.50,VM+=5V, VM≈0V, motor rotational velocity=0.

When α=0.75,VM+=7.5V, VM≈2.5V,the motor rotates normally.

We fixed Vm- to 5v, so that the whole circuit has an up-floating voltage in advance, so that when Vm+ changes from 0 to 10V, we can still obtain the corresponding negative voltage, to realize the motor reversal.

Summary

- Buffering the voltage：To extract the voltage value at a point in the basic circuit without exposing it to the influence of other circuits。We can add an op amp to buffer the output of the resistor network.

- Bidirectional controller：We can introduce a constant potential similar to ground as a comparison value at one end of the target position. When we cannot directly input negative voltage, we can consider comparing the voltage difference between the input end and the constant value to complete the target.